Sensor-Based Simulation (IoT)

Title:

LED ON/OFF using Temperature Sensor in Tinkercad

Aim

To simulate an IoT system where an LED turns ON when temperature increases and turns OFF when temperature decreases.

Components Required

Arduino UNO

TMP36 Temperature Sensor

LED

220Ω Resistor

Breadboard

Jumper Wires

Circuit Connections

TMP36 Sensor

Left Pin → 5V

Middle Pin → A0

Right Pin → GND

LED

Positive leg → Pin 7 through 220Ω resistor

Negative leg → GND

Arduino Code

C++

int sensorPin = A0;

int ledPin = 7;

void setup()

{

pinMode(ledPin, OUTPUT);

Serial.begin(9600);

}

void loop()

{

int reading = analogRead(sensorPin);

float voltage = reading * 5.0;

voltage = voltage / 1024.0;

float temperatureC = (voltage - 0.5) * 100;

Serial.print("Temperature: ");

Serial.println(temperatureC);

if (temperatureC > 30)

{

digitalWrite(ledPin, HIGH);

}

else

{

digitalWrite(ledPin, LOW);

}

delay(1000);

}

Working Explanation

- The TMP36 sensor senses temperature.

- Arduino reads sensor values from analog pin A0.

- Temperature is calculated in Celsius.

- If temperature becomes greater than 30°C:

LED turns ON.

- If temperature is below 30°C:

LED turns OFF.

- Output can also be monitored in Serial Monitor.

Steps in Tinkercad

- Open Tinkercad Circuits.

- Create a new circuit.

- Add Arduino UNO.

- Add TMP36 sensor and LED.

- Make the connections.

- Paste the Arduino code.

- Click “Start Simulation”.

- Change sensor temperature to test LED ON/OFF.

Conclusion

The IoT sensor-based simulation was successfully implemented using Arduino and a temperature sensor in Tinkercad. The LED automatically responds according to temperature changes.

## Simulation Diagram Reference .

{

digitalWrite(ledPin, HIGH);

}

else

{

digitalWrite(ledPin, LOW);

}

delay(1000);

}

# Working Explanation

- The TMP36 sensor senses temperature.

- Arduino reads sensor values from analog pin A0.

- Temperature is calculated in Celsius.

- If temperature becomes greater than 30°C:

  - LED turns ON.

- If temperature is below 30°C:

  - LED turns OFF.

- Output can also be monitored in Serial Monitor.

# Steps in Tinkercad

- Open Tinkercad Circuits.

- Create a new circuit.

- Add Arduino UNO.

- Add TMP36 sensor and LED.

- Make the connections.

- Paste the Arduino code.

- Click “Start Simulation”.

- Change sensor temperature to test LED ON/OFF.

# Conclusion

The IoT sensor-based simulation was successfully implemented using Arduino and a temperature sensor in Tinkercad. The LED automatically responds according to temperature changes.

## Simulation Diagram Reference